DIGITAL SYSTEM DESIGN APPLICATIONS

(CRN: 11275)

THE REPORT OF EXPERIMENT – 2

Faculty of Electrical and Electronics Engineering

Electronics and Communication Engineering

Yusuf Tekin – 040200043

1.Decoder

Decoder Verilog Code

`timescale 1ns / 1ps

module DECODER(

input [3:0] IN,

output reg [15:0] OUT

);

always @(*) begin

case(IN)

4'b0000: OUT = 16'b0000_0000_0000_0001;

4'b0001: OUT = 16'b0000_0000_0000_0010;

4'b0010: OUT = 16'b0000_0000_0000_0100;

4'b0011: OUT = 16'b0000_0000_0000_1000;

4'b0100: OUT = 16'b0000_0000_0001_0000;

4'b0101: OUT = 16'b0000_0000_0010_0000;

4'b0110: OUT = 16'b0000_0000_0100_0000;

4'b0111: OUT = 16'b0000_0000_1000_0000;

4'b1000: OUT = 16'b0000_0001_0000_0000;

4'b1001: OUT = 16'b0000_0010_0000_0000;

4'b1010: OUT = 16'b0000_0100_0000_0000;

4'b1011: OUT = 16'b0000_1000_0000_0000;

4'b1100: OUT = 16'b0001_0000_0000_0000;

4'b1101: OUT = 16'b0010_0000_0000_0000;

4'b1110: OUT = 16'b0100_0000_0000_0000;

4'b1111: OUT = 16'b1000_0000_0000_0000;

default: OUT = 16'b0000_0000_0000_0000;

endcase

end

endmodule

top_module Testbench Code

`timescale 1ns / 1ps

module top_modul_tb();

reg [7:0] sw;

reg [3:0] btn = 4'b0000;

wire [7:0] led;

wire [6:0] cat;

wire [3:0] an;

wire dp;

top_module uut(

.sw(sw),

.btn(btn),

.led(led),

.cat(cat),

.an(an),

.dp(dp)

);

initial begin

sw = 8'b0000_0000; #10;

sw = 8'b0000_0001; #10;

sw = 8'b0000_0010; #10;

sw = 8'b0000_0011; #10;

sw = 8'b0000_0100; #10;

sw = 8'b0000_0101; #10;

sw = 8'b0000_0110; #10;

sw = 8'b0000_0111; #10;

sw = 8'b0000_1000; #10;

sw = 8'b0000_1001; #10;

sw = 8'b0000_1010; #10;

sw = 8'b0000_1011; #10;

sw = 8'b0000_1100; #10;

sw = 8'b0000_1101; #10;

sw = 8'b0000_1110; #10;

sw = 8'b0000_1111; #10;

sw = 8'b0000_0000; #10;

$finish();

end

endmodule

top_module Verilog Code

`timescale 1ns / 1ps

module top_module(

input [7:0] sw,

input [3:0] btn,

output [7:0] led,

output [6:0] cat,

output [3:0] an,

output dp

);

wire [15:0] decoder_out;

wire [3:0] decoder_in;

assign dp = decoder_out[15];

assign cat = decoder_out[14:8];

assign led = decoder_out[7:0];

assign decoder_in[3:0] = sw[3:0];

assign an = 4'b1110;

DECODER decoder1(

.IN(decoder_in),

.OUT(decoder_out)

);

endmodule

Decoder RTL Schematic (at the top_module)

The RTL synthesis tool interprets the pattern of 4-bit addressing as a ROM table where IN is the address and OUT is the data. That is the reason why there is a standalone block of ROM in the RTL Schematic. As it can be seen in the figure, besides the input “btn” all the inputs and outputs are arranged accordingly.

Decoder Behavioral Simulation (at the top_module)

Technology Schematic

Technology Schematic (detailed)

As it can be seen in the detailed Technology Schematic (p.6), there are sixteen LUTs used in the design. The LUTs represents seven “cat”, eight “led” and a “dp” outputs, in total of sixteen outputs. For each four bits value of “sw” input from 0000 to 1111, there is a LUT entry. Since it is required to be unique outputs for each input value, the design is required to be use sixteen LUTs for the decoder.

For instance, if led[0] is desired to be enable, the LUT named led_OBUF(0)_inst_i_1 must has the input values of I[0:3] = {0,0,0,0}. (I = 4’b0000)

Timing Summary - Setup

Timing Summary - Hold

For the largest delay, setup paths are more commonly longer as these represents the delays from register to input or register to register or register to output. In this case there is no difference. It can be seen in the Timing Summary – Setup table the greatest delayed path is the Path 1 which connects sw[0] to cat[1] with the total delay of 10.562.

2. Priority Encoder

Priorty Encoder truthtables, logic statements and schematics

`timescale 1ns / 1ps

module ENCODER(

input [3:0] IN,

output [1:0] OUT,

output V);

assign OUT[0] = IN[3] | (IN[1] & !IN[2]);

assign OUT[1] = IN[2] | IN[3];

assign V = IN[0] | IN[1] | IN[2] | IN[3];

endmodule

Priority Encoder Verilog Code

`timescale 1ns / 1ps

module top_modul_tb();

reg [7:0] sw;

wire [7:0] led;

top_module uut(

.sw(sw),

.btn(btn),

.led(led),

.cat(cat),

.an(an),

.dp(dp)

);

initial begin

sw = 8'b0000_0000; #10;

sw = 8'b0000_0001; #10;

sw = 8'b0000_0010; #10;

sw = 8'b0000_0011; #10;

sw = 8'b0000_0100; #10;

sw = 8'b0000_0101; #10;

sw = 8'b0000_0110; #10;

sw = 8'b0000_0111; #10;

sw = 8'b0000_1000; #10;

sw = 8'b0000_1001; #10;

sw = 8'b0000_1010; #10;

sw = 8'b0000_1011; #10;

sw = 8'b0000_1100; #10;

sw = 8'b0000_1101; #10;

sw = 8'b0000_1110; #10;

sw = 8'b0000_1111; #10;

sw = 8'b0000_0000; #10;

$finish();

end

endmodule

Priorty Encoder Verilog Code top_module

`timescale 1ns / 1ps

module top_module(

input [7:0] sw,

input [3:0] btn,

output [7:0] led,

output [6:0] cat,

output [3:0] an,

output dp

);

wire [1:0] encoder_out;

wire [3:0] encoder_in;

wire v_out;

assign led[1:0] = encoder_out;

assign led[7] = v_out;

assign encoder_in[3:0] = sw[3:0];

ENCODER encoder1(

.IN(encoder_in),

.OUT(encoder_out),

.V(v_out)

);

endmodule

Priority Encoder Testbench Code (top_module)

Priority Encoder RTL Schematic

Priority Encoder Behavioral Simulation

Priority Encoder Post-Synthesis Simulation

Priority Encoder Post-Synthesis Simulation (zoomed in)

In the simulation waves, it is clear that the encoder work as intended, although, there are visible delays at led[0] and led[7] in the post-synthesis simulation. These delays are caused by the different lengths of the paths.

Priority Encoder Technology Schematic

It can be seen in the schematic, after the implementation all the gates are turned into LUTs. “an”, ”cat” and “led[2:6]” outputs created LUTs that are empty. As it can be seen in the technology schematic that the 2-input LUT2 is connected to OUT[1] which makes sense due to the one OR gate it requires to assign. V is a 4-input or process which requires just one 4-input LUT and that also can be seen in the figure. The last LUT that in-use is for the 3-input required assignment OUT[0].

Priority Encoder Setup Timings - Primitive Gates

Priority Encoder Hold Timings - Primitive Gates

Priority Encoder Utility Summary - Primitive Gates

Always – Case Structure:

Priority Encoder Technology Schematic - always/case

The difference of technology schematics between primitive gate design and always/case design is the different paths of the LUT3 block uses. It seems that other than the path there is no practical difference.

Priority Encoder Post-Synthesis Simulation - always/case

1Priority Encoder Post-Synthesis Simulation - always/case (detailed)

The difference of Post-Synthesis Simulations between primitive gate design and always/case design is due to the change on the paths the delay on the led[0] is cancelled.

Priority Encoder Setup Timings - always/case

Priority Encoder Hold Timings - always/case

To observe the differences in terms of timing, it is necessary to inspect both logic delay and net delay separately. In terms of total delay, besides Path 2 and Path 4 all the paths’ delays have increased. For Path 4 the net delay has decreased, and logic delay has stayed the same, that’s why the total delay has decreased. Its same for the Path 2 but Path 2’s logic delay has also decreased. For other paths it can be seen that even though net delays have decreased for most, the logic delays have increased more which results in the increase of the total delay.

Priority Encoder Utility Summary - always/case

3. Multiplexer (MUX)

`timescale 1ns / 1ps

module top_module(

input [7:0] sw,

input [3:0] btn,

output [7:0] led,

output [6:0] cat,

output [3:0] an,

output dp

);

wire [3:0] input_d;

wire [1:0] input_s;

wire output_o;

assign input_d[3:0] = sw[3:0];

assign input_s[1:0] = btn[1:0];

assign led[0] = output_o;

MUX mux1(

.D(input_d),

.S(input_s),

.O(output_o)

);

endmodule

`timescale 1ns / 1ps

module MUX(

input [3:0] D,

input [1:0] S,

output O

);

assign O = (!S[0] & !S[1] & D[0]) + (S[0] & !S[1] & D[1]) + (!S[0] & S[1] & D[2]) + (S[0] & S[1] & D[3]);

endmodule

MUX Verilog Code

MUX Verilog Code - top_module

MUX RTL Schematic

MUX Testbench Code

`timescale 1ns / 1ps

module top_modul_tb();

reg [7:0] sw;

reg [3:0] btn;

wire [7:0] led;

top_module uut(

.sw(sw),

.btn(btn),

.led(led),

.cat(cat),

.an(an),

.dp(dp)

);

initial begin

sw = 8'b0000_0000; btn = 2'b00; #10;

sw = 8'b0000_0001; btn = 2'b00; #10;

sw = 8'b0000_0001; btn = 2'b00; #10;

sw = 8'b0000_0010; btn = 2'b00; #10;

sw = 8'b0000_0010; btn = 2'b01; #10;

sw = 8'b0000_0100; btn = 2'b00; #10;

sw = 8'b0000_0100; btn = 2'b10; #10;

sw = 8'b0000_1000; btn = 2'b00; #10;

sw = 8'b0000_1000; btn = 2'b11; #10;

sw = 8'b0000_0000; btn = 2'b00; #10;

$finish();

end

endmodule

MUX Behavioral Simulation

MUX Technology Schematic

MUX Timing - Setup

MUX Timing - Hold

MUX Utilization Summary

Always/case Structure:

MUX RTL Schematic - always/case

MUX Verilog Code - always/case

`timescale 1ns / 1ps

module MUX(

input [3:0] D,

input [1:0] S,

output reg O

);

always @(*) begin

case(S)

2'b00: O = D[0];

2'b01: O = D[1];

2'b10: O = D[2];

2'b11: O = D[3];

endcase

end

endmodule

MUX Technology Schematic  -  always/case

MUX Setup Timing - always/case

MUX Setup Timing - always/case

MUX Utilization Summary - always/case

Comparing the two different methods’ timing and utilization, there is no difference in terms of utilization and the delays are increased significantly in the always/case method. To be specific, almost all the logic delays stayed same for both Path 1 and Path 2 however, net delay of the Path 1 has increased 251% and for Path 2 158%. Which caused increases in total delays.

4. Demultiplexer (DEMUX)

DEMUX Verilog Code

`timescale 1ns / 1ps

module DEMUX(

input D,

input [1:0] S,

output [3:0] O

);

wire enable_O0, enable_O1, enable_O2, enable_O3;

// AND and NOT gates:

assign enable_O0 = !S[0] & !S[1];

assign enable_O1 = S[0] & !S[1];

assign enable_O2 = !S[0] & S[1];

assign enable_O3 = S[0] & S[1];

//TRI gates:

assign O[0] = enable_O0 ? D : 1'bz;

assign O[1] = enable_O1 ? D : 1'bz;

assign O[2] = enable_O2 ? D : 1'bz;

assign O[3] = enable_O3 ? D : 1'bz;

endmodule

DEMUX Verilog Code - top_module

`timescale 1ns / 1ps

module top_module(

input [7:0] sw,

input [3:0] btn,

output [7:0] led,

output [6:0] cat,

output [3:0] an,

output dp

);

wire input_d;

wire [1:0] input_s;

wire [3:0] output_o;

assign input_d = sw[0];

assign input_s[1:0] = btn[1:0];

assign led[3:0] = output_o[3:0];

DEMUX demux1(

.D(input_d),

.S(input_s),

.O(output_o)

);

endmodule

DEMUX RTL Schematic

DEMUX Testbench Code

`timescale 1ns / 1ps

module top_modul_tb();

reg [7:0] sw;

reg [3:0] btn;

wire [7:0] led;

top_module uut(

.sw(sw),

.btn(btn),

.led(led),

.cat(cat),

.an(an),

.dp(dp)

);

initial begin

sw = 8'b0000_0000; btn = 2'b00; #10;

sw = 8'b0000_0001; btn = 2'b00; #10;

sw = 8'b0000_0000; btn = 2'b01; #10;

sw = 8'b0000_0001; btn = 2'b01; #10;

sw = 8'b0000_0000; btn = 2'b10; #10;

sw = 8'b0000_0001; btn = 2'b10; #10;

sw = 8'b0000_0000; btn = 2'b11; #10;

sw = 8'b0000_0001; btn = 2'b11; #10;

sw = 8'b0000_0000; btn = 2'b00; #10;

$finish();

end

endmodule

DEMUX Behavioral Simulation

DEMUX Technology Schematic - Active Outputs

5. Research

Latches: Latches are memory elements that stores a single bit of information. D-Latches are the most common latches in the FPGA but there are also SR, JK and T latch types. In HDL design the latches are almost always refers to an error in the design due to lack of field of operation it required. A latch does not need a clock to operate which means a latch might work out of time. This creates a possibility of circuit break-down in the process so, the designers should avoid implementing a design with latches in it.1

Leading Zero Counters: It is a circuit that commonly used for floating-point arithmetic, data compressing and digital signal processing. The circuit counts the zeroes of a binary code until the first countered one.2

To make such a circuit, it is common to use a priority encoder which is a digital circuit that takes multiple inputs to output the highest-priority active input (1 in binary number). Let say there is a binary number with the total bit length of N. After finding the highest-priority one with a position of K with a priority encoder, leading zero count can be found by Z = N – K – 1 expression where the Z is the number of zeroes counted.

Fan-in and fan-out: Fan-in refers to the number of inputs a gate can accept and fan-out refers to the number of outputs a gate can drive. In practice a fan-in limitation is use for keeping the delays manageable and avoid excessive power consumption. Fan-out in practical use aims to ensure the reliability and avoid unnecessary power consumption.3

References

- Vivado Design Suite User Guide: Synthesis (UG901), Chapter: “HDL Coding Techniques, Flip-Flops, Registers and Latches”, Chapter: “Latches”

- https://digitalsystemdesign.in/leading-zero-counter/?srsltid=AfmBOorDdQog8Ro7shAFHPwplmMKFvdikEHBwGfNL6NohjjQh6sXHzsU#google_vignette

- https://vhdlwhiz.com/terminology/fan-out/